A Few Useful Energy Quantities

It is important to remember that energy is measured in joules, and power is measured in watts. One joule = 1 watt ´ 1 second. People usually associate the watt with electricity, but that is because the SI system has traditionally been used only for electricity in the US. The watt is a general unit for power --- energy divided by time --- regardless of whether electricity is involved.

The so-called power company sends you a bill for energy (not power), but it uses a non-SI unit for time. To get the energy, they multiply kilowatts by hours to get kWh.

Efficiency

The term efficiency is often misused, so we need to define it carefully. A machine converts energy from one form into another, but in doing so "wastes" some energy. For example, an automobile engine might burn enough gasoline to produce 10,000 J, but only 2500 J results in mechanical energy, and the remaining 7500 goes out as hot exhaust through the tailpipe or as heat through the radiator. The efficiency is the ratio of the delivered energy (the 2500 J) to the input energy (10,000 J). That is, eff = 2500/10,000 = 0.25, or 25% for this case.

The largest efficiency possible is 1.0, or 100%. ("One-hundred ten percent" is an ahtlete's expression used to sell deodorant.)

Steam engines, gasoline, engines, diesel engines, turbine engines and the lot all have the purpose of converting heat into mechanical energy. The efficiency of heat engines is notoriously low, but not because the engineering is poor. Inevitably, they must throw away heat, and their waste heat must be hotter than the environment (lest the heat not leave the engine.)

Converting electric energy to mechanical energy (using motors) or mechanical energy to electricity (using generators) is often very efficient --- perhaps 90-95%.

Converting sunlight to electricity is not easy, for various technical reasons. There will always be energy losses that become heat. The only photocells available for large-scale applications are about 10% efficient. Some recent photocells made out of exotic materials have efficiencies approaching 18-19%. They are very costly, and very small.

Some Energy And Power Values

Units and equations are nice, but rather meaningless without any numbers to go into them. In this section, I give a few very approximate values for various sources of energy and for various uses.

The Heat Content table shows the heat that is released when various fuels are burned. By "wood," I refer to most wood (ash, oak, pine ...). A log of oak releases more heat than one of pine simply because it is heavier, not because the wood is different. Similarly, a kilogram of petroleum --- whether it be gasoline, diesel fuel, or crude oil --- releases the same amount of heat. However, crude oil is somewhat denser, so a bucketful of crude oil will release more heat than a bucketful of gasoline.

Coal always contains mineral impurities. Very poor coal has about 50% impurities, so when you "burn" a kilogram of very poor coal, you are really burning about a half kilogram of "real" coal; the rest is merely weight that becomes ash.

Methane is the major component of natural gas.

There is no natural supply of hydrogen, but hydrogen could be a manufactured fuel, made by removing it from water. It takes a lot of energy to do so, of course, but at least the hydrogen can be transported.

Energy & Power Usage

Perhaps you view energy in personal terms (for example, how much gasoline and fuel oil you use in a year), and perhaps you think of the State instead. You may be worried about how much energy is used annually by the US or by the world. Here are some approximate numbers for you.